TECHNICAL SPECIFICATIONS

for Digital Microwave Radio Relay Systems

| Remarks : | All data in this sheet are the guarantied values Supplementary information and the information indicated *) are not obligatory for type - approval of the radio equipment |
| --- | --- |

| Type |  | DTL 2001X PCM, DTL 10G PCM |
| --- | --- | --- |
| Producer |  | Phobos Engineering s.r.o. (former Phobos Audio) |
| Transmission capacity | Mbit/s | 2 |
| Frequency band | MHz | 10 300 – 10 600 |
| Modulation method |  | FSK |
| Radio frequency data |  |  |
| Equipment works according to added |  |  |
| radio frequency channel arrangements | Annex No. | GP-02/94 |
| Channel spacing | MHz | 14 |
| By the connection to one antenna |  |  |
| - in the same polarisation |  |  |
| min. channels spacing | MHz | 112 |
| duplex spacing | MHz | SIMPLEX |
| min. Tx – Rx spacing | MHz | 28 |
| max. number of channels |  | 15 |
| - in the cross - polarisation |  |  |
| min. channels spacing | MHz | ---------- |
| duplex spacing | MHz | ---------- |
| min. Tx – Rx spacing | MHz | ---------- |
| max. number of channels |  | ---------- |

| By the connection to different antennas (in the same direction) |  |  |
| --- | --- | --- |
| - in the same polarisation |  |  |
| min. channels spacing | MHz | 14 |
| duplex spacing | MHz | ---------- |
| min. Tx – Rx spacing | MHz | 28 |
| - in the cross – polarisation |  |  |
| min. channels spacing | MHz | ---------- |
| duplex spacing | MHz | ---------- |
| min. Tx – Rx spacing | MHz | ---------- |
| Transmitted data |  |  |
| Transmission capacity: |  |  |
| main signal | Mbit/s | 2 |
| gross bit rate | Mbit/s | ---------- |
| additional transmission (way side traffic) |  |  |
| digital | Mbit/s | ---------- |
| customer access | kbit/s | ---------- |
| asynchronous data | kbit/s | 9,6 |
| analogue |  | ---------- |
| Forward error correction method |  | ---------- |
| Possibilities of system configurations |  |  |
| Frequency diversity (max. configuration) |  | ---------- |
| Hot standby (HSB) |  | ---------- |
| Space diversity |  | ---------- |
| Cross polarisation interference canceller (XPIC) |  | ---------- |

Definition of measured points

| > | Z´ | Transmitter | A´ | Transmitter | B´ | Branching | C´ | Waveguide | D´ |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | filter |  | circuit |  |  |  |  |
|  | D | Waveguide | C | Branching | B | Receiver | A | Receiver | Z | > |
|  |  |  |  | circuit |  | filter |  |  |  |  |

Please, cross out no relevant measured points

| Transmitter |  |  |
| --- | --- | --- |
| Output power ( C´) | dBm | +3 |
| ATPC dynamic range | dB | ---------- |
| Transmitter frequency stability | x 10-6 | +/- 100 |
| L.O. frequency stability | x 10-6 | +/- 100 |
| IF frequency | MHz | ---------- |
| Guarantee transmit frequency spectrum mask | Annex No. | ---------- |
| *) Amplitude- frequency response of RF filter | Annex No. | ---------- |
| Spurious emissions Max. level in the frequency band ( C´ ) |  |  |
| 30 MHz to 21.2 GHz | dBm | -50 |
| 21.2 GHz to 55 GHz | dBm | -30 |
| Receiver |  |  |
| L.O. frequency stability | x 10-6 | +/- 100 |
| IF frequency | MHz | 470 |
| Noise figure ( C ) | dB | 1.5 |
| Max. input level for BER= 1x10 – 9 ( C ) | dBm | -10 |
| Threshold level for BER= 1.10- 6 ( C ) | dBm | -82 |
| Threshold level for BER= 1. 10- 3 ( C ) | dBm | -85 |
| BER range at the switch - on AIS gen. |  | ---------- |
| Thermal noise level ( C ) | dBm | -101.0 |
| Noise bandwidth | MHz | 14 |
| Carrier-to-noise ratio for BER= 1x10 -6 ( A ) ( C ) | dB | 19 |
| Carrier-to-noise ratio for BER= 1x10 -3 ( A ) ( C ) | dB | 16 |
| *) Amplitude- frequency response of IF filter | Fig. | ---------- |
| *) Amplitude- frequency response of RF filter | Fig. | ---------- |
| Spurious emissions ( C ) Max. level in the frequency band |  |  |
| 30 MHz to 21.2 GHz | dBm | -50 |
| 21.2 GHz to 55 GHz | dBm | -30 |
| Threshold level degradation max. 1 dB for CW spurious interference (C/I=-30dB) for frequencies from 30 MHz to 55 GHz |  |  |
| excluding the frequency band fc  | MHz | 28 |
| Net Frequency Discrimination (NFD) factor, Interference Reduction Factor (IRF) |  |  |
| - half channel frequency spacing | dB | ---------- |
| - one channel frequency spacing | dB | ---------- |
| - 1.5 channel frequency spacing | dB | ---------- |
| - two times channel frequency spacing | dB | ---------- |
| Signature for BER= 10-6 and  =6.3 ns |  |  |
| - minimum phase | Annex No. | ---------- |
| - non- minimum phase | Annex No. | ---------- |
| Signature for BER= 10-3 and  =6.3 ns |  |  |
| - minimum phase | Annex No. | ---------- |
| - non- minimum phase | Annex No. | ---------- |

| XPIC |  |  |
| --- | --- | --- |
| Attenuation of cross polarisation interference | dB | ---------- |
| Microwave elements loss | dB | ---------- |
| All relevant data which are changed in the configuration with XPIC |  |  |
|  |  | ---------- |
|  |  | ---------- |
|  |  | ---------- |
|  |  | ---------- |
|  |  | ---------- |
|  |  | ---------- |
|  |  | ---------- |
|  |  | ---------- |
| Analog interface (points Z' and Z) |  |  |
| Annex |  | 1 |
| Parameters according to | ITU-T Rec | ---------- |
| Impedance |  |  |
| asymmetrical symmetrical |  | ---------- |
| Jitter and wander characteristics |  |  |
| - maximum permissible jitter at input port | acc. to | ---------- |
| -maximum output jitter | acc. to | ---------- |
| - jitter transfer characteristic | acc. to | ---------- |

| Protection switching system |  |  |  |  |
| --- | --- | --- | --- | --- |
| Hot standby |  | NO |  |  |
| - interruption time at the change over |  |  |  |  |
| Channel configuration 1+1 |  | NO |  |  |
| Channel configuration N+1 |  | NO |  |  |
| - interruption time at the change over |  | ---------- |  |  |
| - hitless |  | NO |  |  |
| range of time delay compensation |  |  |  |  |
| - static | bit | ---------- |  |  |
| - dynamic | bit | ---------- |  |  |
| Environmental conditions |  |  |  |  |
| Temperature range |  |  |  |  |
| indoor equipment |  |  |  |  |
| - guarantee of performance | oC | 0 to +45 |  |  |
| - operation possible | oC | -10 to +45 |  |  |
| outdoor equipment |  |  |  |  |
| - guarantee of performance | oC | -30 to +50 |  |  |
| - operation possible | oC | -35 to +55 |  |  |
| Max. wind velocity (elastic deformation, change |  |  |  |  |
| of gain of the antenna max. 3 dB) | m/s | 30 |  |  |
| Power supply |  |  |  |  |
| Voltage range | V | 230 V, 50Hz |  |  |
| Earth pole |  | ---------- |  |  |
| Max. ripple voltage | mVp-p | ---------- |  |  |
| Limits of radio interference characteristics of equipment |  |  |  |  |
| According to standard |  | EN 55 022 Class B like IEC CISPR 22 |  |  |
| Susceptibility to Electromagnetic and Electrostatic Field |  |  |  |  |
| Radiated electromagnetic field requirements |  |  |  |  |
| according to standard |  | EN 61000-4-3 |  |  |
| Electrostatic discharge requirements |  |  |  |  |
| according to standard |  | EN 61000-4-2 |  |  |
| Safety requirements for human protection |  |  |  |  |
| according to standard |  | EN 60 950 like IEC 950 |  |  |
| Microwave elements branching circuit loss |  |  |  |  |
| Transmitter filter | dB | ---------- |  |  |
| Receiver filter | dB | ---------- |  |  |
| Circulator (1 transit) | dB | ---------- |  |  |
| Next microwave elements: |  |  |  |  |
|  | dB | ---------- |  |  |
|  | dB | ---------- |  |  |
|  | dB | ---------- |  |  |
| Overall attenuation on transmit side |  |  |  |  |
| in configuration |  | 1+0 | 1+1 | 1+1HSB |
| between points A' and C' | dB | 0 | N.A. | N.A. |
| (for configuration 1+0 is the value 0, if Tx and Rx |  |  |  |  |
| parameters are defined in the point C´) |  |  |  |  |
| Overall attenuation on receive side |  |  |  |  |
| in configuration |  | 1+0 | 1+1 | 1+1HSB |
| between points C and A | dB | 0 | N.A. | N.A. |
| (for configuration 1+0 is the value 0, if Tx and Rx |  |  |  |  |
| parameters are defined in the point C) |  |  |  |  |
| VSWR of branching network (waveguide side) |  | ---------- |  |  |
| *)type of branching network |  |  |  |  |
| flange(waveguide side) |  | ---------- |  |  |
| Waveguide |  |  |  |  |
| type |  | ---------- |  |  |
| producer |  | ---------- |  |  |
| attenuation | dB/m | ---------- |  |  |
| VSWR |  | ---------- |  |  |

| Antennas (TX,RX units in focus points) |  |  |
| --- | --- | --- |
| Type |  | MA 35 |
| Producer |  | PHOBOS |
| Diameter | m | 0.35 |
| Gain | dBi | 29 |
| Cross polarisation discrimination | dB | ---------- |
| Radiation pattern envelope |  |  |
| -in the same polarisation |  | GP-02/94 |
| -in the cross polarisation | Annex No. | ---------- |
| VSWR |  | ---------- |
| *) Type of flange |  | ---------- |
| Antennas |  |  |
| Type |  | MA 65 |
| Producer |  | PHOBOS |
| Diameter | m | 0.65 |
| Gain | dBi | 34 |
| Cross polarisation discrimination | dB | ---------- |
| Radiation pattern envelope |  |  |
| -in the same polarisation |  | GL |
| -in the cross polarisation | Annex No. | ---------- |
| VSWR |  | ---------- |
| *) Type of flange |  | ---------- |

| Antennas |  |  |
| --- | --- | --- |
| Type |  | MA 90 |
| Producer |  | PHOBOS |
| Diameter | m | 0.90 |
| Gain | dBi | 37 |
| Cross polarisation discrimination | dB | ---------- |
| Radiation pattern envelope |  |  |
| -in the same polarisation |  | GL |
| -in the cross polarisation | Annex No. | ---------- |
| VSWR |  | ---------- |
| *) Type of flange |  | ---------- |
| Antennas |  |  |
| Type |  | MA 120 |
| Producer |  | PHOBOS |
| Diameter | m | 1.2 |
| Gain | dBi | 40 |
| Cross polarisation discrimination | dB | ---------- |
| Radiation pattern envelope |  |  |
| -in the same polarisation |  | GL |
| -in the cross polarisation | Annex No. | ---------- |
| VSWR |  | ---------- |
| *) Type of flange |  | ---------- |

Supplementary information

| Power consumption |  |  |
| --- | --- | --- |
| Complete transmitter | W | 12 |
| Complete receiver | W | 18 |
| Protection switching | W | ---------- |
| Service channel | W | ---------- |
| Supervisory equipment | W | ---------- |
| MTBF |  |  |
| Complete transmitter | h | > 25 000 |
| Complete receiver | h | > 25 000 |
| Protection switching | h | ---------- |
| Service channel | h | > 25 000 |
| Supervisory equipment | h | ---------- |
| Supervisory and Control Equipment |  |  |
| Max. number of supervised stations |  | ---------- |
| Max. number of messages on selected station |  | ---------- |
| Max. number of commands on selected station |  | ---------- |
| Max. number of external messages on selec. st. |  | ---------- |

| Type and quantity of combustible materials |  |  |
| --- | --- | --- |
| Typical individual rack (construction unit) contents |  |  |
| Type of materials |  | Quantity [g] |
| All combustive materials less than |  | 100 g |
| Number of annexes: 1 |  |  |
| Date: 15.02.2000 |  |  |

Annex 1:

Analog interface:

# Frequency range:								5Hz – 59 kHz (+0.3, -0.7dB)

Nominal input level:								+ 6 dBu

Nominal output level:								+ 6 dBu

Input impedance (input is unbalanced):				10 kohm

Output impedance (output is unbalanced):				75 ohm

S/N:											more then 72 dB